EZScope 1.0 - Digital oscilloscope and spectroscope

---------------------------------------------------

The EZScope is a digital oscilloscope, digital tone generator,

and digital spectroscope.  The EZScope requires the use of

Analog Devices' ADSP 2181 EZ-KIT as its input and output.  The

Ezscope can measure system response by stepping its tone

generator.  The EZScope runs well on HP200LX (at least for me.)

Features:

---------

o   32 k/Sec digital oscilloscope

o   digital spectroscope with frequency resolution of 320 Hz

o   user selectable digital sine wave generator output level

and frequency

o   pass through/tone generator/chirp mode

o   system response measurement; the number of frequency points

is selectable

o   unfiltered decimation of sampled data

o   realtime FFT spectrogram and with memory for highest level

o   frequency read out of major frequency component.

o   simultaneous display of time domain and frequency domain

data

Legal Notices

-------------

In the following paragraphs, the terms 'the software,' 'this

software,' and 'it' refer to EZScope.  The author refers to

me, Alfred Lee.  I may be contacted at the Internet address

'alfred1520@geocities.com'.  In the event that I am not

reachable at this address, please conduct a search on the any

popular Internet search engine for 'alfred1520'.

Copyrights

----------

This software is copyrighted Freeware.  World wide copyrights

are reserved by the author.

The author also reserves the rights to re-issue the software

as Shareware or as commercial software.  The Freeware status

of this software shall terminate at such time.  You are then

requested to acquire an appropriate license to continue using

the software.

Distribution

------------

Since this software is currently being released as Freeware,

you may re-distribute this software subjecting to each and

all of the following conditions:

1.  This document must be present.

2.  All files are distributed together as a single package.

3.  All files remain in their original form without any

modification.

4.  The re-distribution will result in absolutely no direct

or indirect financial gains for all of the parties

involved.

5.  The re-distribution is non-commercial.

The followings are strictly prohibited without explicit

written permission from the author:

1.  Inclusion of any part or the whole of this program in any

periodical, book, or any other publication, or bundling

with any other package or product, or adding it to any

software collection for any direct or indirect commercial

gains or any other commercial purposes or promotions.

Distribution with no possibility of direct or indirect

financial gain is permitted.

2.  Inclusion of any part or the whole of this program on any

Internet Wed site if such a site is commercially oriented

and if any financial gain can be derived from such an

inclusion.  Distribution with no possibility of direct or

indirect financial gains is permitted.

3.  Charging a fee for re-distribution.

4.  Implying or referencing to this software in an attempt to

enhance the commercial value of any commercial product for

financial gain or commercial promotion.

5.  Using this software in an educational or teaching

institution that charges a fee or tuition.  Non-profit

institutions that do not charge a fee nor tuition or only

charge a minimal material fee are exempted.

Any exception requires the explicit written permission form

the author and perhaps compensation to the author.

A special permission is hereby granted to the Analog Devices

and its agents to freely distribute this software as long as

no fee is charged directly nor indirectly.  This is in

appreciation of their producing the low cost EZ-KIT for the

general DSP community.

Warranty

--------

This software is provided as is without any warranty,

expressed or implied.  This includes without limitation the

fitness of purpose or application.

This software has not been extensively tested.  It has only

been tested by a single user (the author) on a particular

computer (HP200LX.)  It is never implied that the program will

function properly and harmlessly on a different machine due to

interaction between this program and any and all other

installed components on your particular computer.

The author shall not be liable for any damages, including any

lost profits, whether direct, indirect, special, incidental,

or consequential arising from a failure of this program or

accompanying files to operate in a manner desired by the user.

The author shall not be liable for any damage to data or

property which may be caused directly or indirectly by the use

of this program.  You shall bear the all the risks that may

arrise out of the use of this software.

This software may use unproven software routines to handle

the serial port communication.  Please use the documented

method to exit the software.  Failure to following this

instruction will result in the Interrupt Vector not being

properly restored and causes the system to lock up and may

result in file system corruption or loss of data.  Backup you

data.

Using EZScope

-------------

A DSP program must be downloaded to the EZ-KIT before the

EZScope can be used.  For your convenience, EZLD4.ZIP, which is

available from ftp.analog.com, is included in this

distribution.  Please extract EZLD.COM.

A batch file, scope.bat, can be used to automatically download

the DSP program and start EZScope.  Please be sure to modify

for your comm port setting if you are not using com1.

Command line argument:

----------------------

There is only one optional command line argument:

ezscope [1 | 2 | 3 | 4]

It specifies the comm port, e.g.

ezscope 2

for comm port 2.

Keyboard commands:

------------------

Press the indicated key has the described effect.

any other key

quit program.

' ' space

Update tone generator frequency.

'?'

'h'

'H'

Help screen.

<F1>

freq++

Increase the current frequency by 41%.

<F2>

freq--

Decrease the current frequency by 39%.

<F3>

level++

Increase the current output level by 6 dB.

<F4>

level--

Decrease the current output level by 6 dB.

<F5>

scoFFT

Cycles between oscilloscope, spectroscope,

spectroscope with memory for highest recorded level,

and split screen of the first 256 points of the scope

and the 256 points from the 512 points spectroscope

data (higher of the two adjacent points are displayed.)

<F6>

freq =

Set output frequency.

<F7>

level=

Set output level attenuation.  Enter attenuation

in dB (positive value.)

<F8>

pts/swp

Sets the number of steps per sweep and start the

frequency sweep for system response measurement.  It

steps the sine wave generator between 40 Hz and

15940 Hz in the given number of steps.  For each step,

the highest amplitude of the system response is

recorded.  Press any key to proceed after the system

response curve is plotted.

<F9>

decima

Sets the decimation ratio for the effect of lower

sampling rate.  No low pass filtering is performed so

beware of aliasing.

<F10>

lp/gen

Cycles between digital sine wave generator, chirp, and

analog pass through.  The chirp combined with

spectroscope gives a quick and literally dirty look

at the system response.

A side effect of the chirp generator is that the

overall system delay becomes apparent.  This delay

includes the DSP serial communication delay and the

CODEC delay.  A delay of 37 samples should eliminate

the EZ-KIT system delay for a direct looping back from

the output to the input.  This delay can be changed by

the '+', '-', '*', and '/' keys.  You can use side

effect to measure system delay.

'+'

Increase the delay by 1.

'-'

Decrease the delay by 1.

'*'

Increase the delay by 15.

'/'

Decrease the delay by 15.

Other feature:

--------------

After the sp.exe has been dowlnloaded and running but before

EZScope is started or after the EZScope is exited, the FL1 LED

will be blinking at a rate depended on the input signal level.

The higher the input level, the faster the blink rate.

A Guided Tour

-------------

You need the EZ-KIT, its power supply, RS-232 cable, and a

loopback cable to connect the EZ-KIT's output jack back to its

input jack.  Make sure that the EZ-KIT has been properly

powered up, the loopback cable is connected, and the RS-232

cable is installed.

1.  Reset the EZ-KIT and wait for it to complete its

initialization.

2.  Run the batch file scope.bat.  Wait for the download to

complete as described in scope.bat.

3.  Press the space bar to continue.  The EZScope should

display a sine wave.

4.  Press <F9> and enter 20.  Notice that the output frequency

changes automatically.  When the display finally stop, the

system response is plotted.

5.  Press <F1>, <F2>, <F3>, or <F4>, to change the level and

frequency.

6.  Press <F5> several times.  Each time wait for a moment to

see the display format changing to a new one.  Eventually

you should get back to the original format.

7.  Try <F6> and <F7> to set new output level and frequency.

8.  Press <F6> and set the frequency to 40 Hz.  Notice that

the display contains on a few complete cycles.  Press <F9>

and set the decimation factor to 5.  Notice that you can

now see more sysles.

9.  Be sure you are in scope display.  Press <F10> several

times.  Each time notice that the wave displaay changes.

10. Press <F10> until you see the chirp signal.  Notice the

steady chirp signal starts at 30+ sample.  This is the

system delay.  Press '/', '*', '-', and '+', to change the

system delay.

Please excuse the inadequate description of various DSP terms

in clear terms for the DSP novices.

-------------------------

Please send comments, suggestions, bug reports to

Internet: alfred1520@geocities.com

(if this address becomes obsolete, please search for

alfred1520 on Internet search engines.)

so that I can consider them for a future release.

Alfred Lee, Mar 14, 1998

Change History:

---------------

98/03/14 - Version 1.0, the original release

--------------------------

Information to be ignored:

swept.dat is generated using DSPworks 'Generating Swept Sine

Wave': 40 to 15940 Hz, 32000 kHz fs, 1024 points 1 sweep

angular phase delay 32.668 degree, zero to peak 1, DC offset 0

Swept type linear