���������������������������������������������������������������

MODEM

(c)2001 Stefan Peichl, Heidelberg, Germany

http://home.t-online.de/home/stefan.peichl/

eMail: stefan.peichl@t-online.de

���������������������������������������������������������������

MODEM.COM detects modems and UARTs at a specified or all ports.

If you call MODEM with a comport number in the range 1-4,

only this comport is checked for a connected modem. It returns

Errorlevel 0 if a modem was found or 1 if no modem was found.

If you call MODEM without any commandline parameter, all four

comports are checked for connected modems, and Errorlevel 0

is returned, if at least one modem was found. If no modem was

found at all, Errorlevel 1 is returned.

MODEM checks for the CTS signal. That is, an external modem is

only detected, if it is powered.

Additionally MODEM reports the base address of the port, the

used IRQ level and the UART type of the comport. The following

chips are detected:

Chip          Description

------    ----------------------

8250     no FIFO, no ScratchReg

16450     no FIFO

16550     no working FIFO

16550A    working FIFO

The typical output of MODEM called on a palmtop with an external

modem connected to COM1 and a PCMCIA modem in COM2 looks like:

COM1 � 03F8 � IRQ4 � 16450  UART (no FIFO) � Modem found

COM2 � 02F8 � IRQ? � 16550A UART with FIFO � Modem found

COM3 � N/A

COM4 � N/A

You may also use MODEM to initialize the base addresses of all

4 comports with the standard values by issuing

MODEM i

the displayed result is:

Init � COM1=03F8 � COM2=02F8 � COM3=03E8 � COM4=02E8

I found this useful to get access to the comports in a Win95

DOS box.

MODEM ? displays the help screen.

You may use MODEM in a batch file in order to setup a software

according to the actual connected hardware.

See SETCOM.BAT for a simple example on how to use MODEM.COM

together with WWW/LX and WWWSET. For an automatic setup of the

correct comport you only have to put the line

PreOnline=SETCOM.BAT

into the [SYSTEM] section of POST.CFG